Daily Excavation Inspection Checklist

For Free Safety & Health Consultation Assistance:

https://www.doli.virginia.gov/vosh-programs/consultation/

| Project Name: | Date / Time: |
| --- | --- |
| Competent Person: | Weather: |
| Soil Type: |  |
| Excavation Depth x Width x Length: |  |
| Protection System Used: |  |
| Activities In Excavation: |  |

| Yes | No | N/A | Excavation |
| --- | --- | --- | --- |
|  |  |  | Excavations & Protective Systems inspected by Competent Person daily, before start of work. |
|  |  |  | Competent Person has authority to remove workers from excavation immediately. |
|  |  |  | Surface encumbrances supported or removed. |
|  |  |  | Employees protected from loose rock or soil. |
|  |  |  | Hard hats worn by all employees. |
|  |  |  | Spoils, materials, and equipment set back a minimum of 2' from edge of excavation. |
|  |  |  | Barriers provided at all remote excavations, wells, pits, shafts, etc. |
|  |  |  | Walkways and bridges over excavations 6' or more in depth equipped with guardrails. |
|  |  |  | Warning vests or other highly visible PPE provided and worn by all employees exposed to vehicular traffic. |
|  |  |  | Employees prohibited from working or walking under suspended loads. |
|  |  |  | Employees prohibited from working on faces of sloped or benched excavations above other employees. |
|  |  |  | Warning system established and used when mobile equipment is operating near edge of excavation. |
| Yes | No | N/A | Utilities |
|  |  |  | Utility companies contacted and/or utilities located. |
|  |  |  | Exact location of utilities marked when near excavation. |
|  |  |  | Underground installations protected, supported, or removed when excavation is open. |
| Yes | No | N/A | Wet Conditions |
|  |  |  | Precautions taken to protect employees from accumulation of water. |
|  |  |  | Water removal equipment monitored by Competent Person. |
|  |  |  | Surface water controlled or diverted. |
|  |  |  | Inspection made after each rainstorm. |
| Yes | No | N/A | Hazardous Atmosphere |
|  |  |  | Atmosphere tested when there is a possibility of oxygen deficiency or build-up of hazardous gases. |
|  |  |  | Oxygen content is between 19.5% and 21%. |
|  |  |  | Ventilation provided to prevent flammable gas build-up to 20% of lower explosive limit of the gas. |
|  |  |  | Testing conducted to ensure that atmosphere remains safe. |
|  |  |  | Emergency Response Equipment readily available where a hazardous atmosphere could or does exist. |
|  |  |  | Employees trained in the use of Personal Protective and Emergency Response Equipment. |
|  |  |  | Safety harness and life line individually attended when employees enter deep confined excavation. |